車輛動態與控制

核心能力一  轉移函數之控制系統

- 何謂轉移函數?

T(s)=Y(s)U(s)⇒Y(s)=T(s)⋅U(s)

- 何謂頻率響應?

Δ frequency Response：系統輸入為 正弦波      討論的Frequency輸出

Consider a linear time–invariant stable system G(S).

If the input is Asinwt, then the steady- state output is Bsin(wt+ϕ) where

B=AG(jw) , ϕ=∠G(jw)=tan−1G(jw)的實部G(jw)的虛部

Thus G(jw)=G(jw)∠G(jw)

複數表示法         相量表示法

- 何謂波德圖?

Δ Bode Plot  波德圖

T(jw)=T(jw)∠T(jw)≡M∠φ

As different from polar plot, Bode plots are sketched in two parts, namely the magnitude part and the phase part.

- Magnitude of G(jw) is drawn versus the frequency w

For convenience, the magnitude is plotted based on the logarithmic operator

(取log是為了方便相加)

∵ G1(jw)G2(jw)⋯Gn(jw)=G1(jw)G2(jw)⋯Gn(jw)

if we take logarithmic operation , then logG1G2⋯Gn=logG1+logG2+⋯+logGn

which can be obtained by adding all the individual logGi together

- Phase of T(jw)=T(jw)∠T(jw),φ=∠T(jw)

If G(jw)=G1(jw)G2(jw)⋯Gn(jw)

⇒∠G(jw)=∠G1(jw)+∠G2(jw)+⋯+∠Gn(jw)

By adding all∠Gi(jw) together, ∠G(jw) is then obtained.

- 比例、微分、積分控制器之形式及特性為何?

控制器分類

R      E                       Y

GC(s)

G(s)

+

-

- 比例控制(proportional control)

G(s)=KP

特性：

- 改善穩定度

- 改善穩態誤差

- 降低靈敏度

- 抑制雜訊

- 微分控制 (derivative control)

G(s)=KDS

特性：

- 改善穩定度

- 提升響應速度

- 增加雜寬

- 放大雜訊 (x)

- 積分控制 (integral control)

G(s)=KIS

特性：

- 增加系統type消除穩態誤差

- 降低響應速度 (x)

- 易造成系統不穩定 (x)

- 比例+微分 (PD)

G(s)=KP+KDS

- 比例+積分 (PI)

G(s)=KP+KIS

- 比例+積分+微分 (PID)

G(s)=KP+KDS+KIS

例題 1:

When the system shown in the Figure 1-(a) is subject to a unit step input the

system output responds as shown in Figure 1－(b) Determine the values of T and K from the response curve.

Figure 1-(a)

例題 2:

Estimate the transfer for the” Bode plot” for the magnitude shown in the figure. Assume it is minimum phase.

G(s)=Ks+2s+2s+10s+30

G(0)=0.2K2×10×30=1			∴ K=3000

∵20log1=0dB

s=1

∴ G(s)=3000s+0.2s+2s+10s+30

例題 3:

A mechanical system is shown in Figure. The motion of mass M is small and is constrained by a spring with constant k and a damper with coefficient c. A rigid bar is used to connect the mass and the disk. The motion of the disk is constrained by a torsional spring with spring constant G. The inertia of the disk is J and the friction of the disk is negligible. Derive the transfer function of this system in which y is the output and f(t) is the input.

Sol:

The free body diagrams are shown below

The dynamic equation

f−p−ky−cy=My..... (1)

pr−Gθ=Jθ .............. (2)

Because of θ=yr

(2)⇒p=Jyr2+Gyr2 into (1)

⇒(M+Jr2)y+cy+(K+Gr2)y=f

Taking Laplace transforms and we can get the transfer function of the system

Y(s)F(s)=1(M+Jr2)s2+cs+(K+Gr2)

例題 4:

例：Consider a system shown in Figure. Suppose r(t)=sin(t+60∘)−2cos(2t−30∘) Find the steady- state response y(t)

重疊定理響應

Sol：

G(S)=Y(s)R(s)=10s+11+10s+1=10s+11

(1) r(t)=sin(t+60∘),w=1       G(j1)=10j1+11=10122

∠G(j1)=−tan−1111                虛         實

∴y(t)=10122sin(t+60∘−tan−1111)

(2) r(t)=−2cos(2t−30∘),w=2

G(j2)=10j2+11=10125=25

∠G(j2)=−tan−1211

∴y(t)=−45cos(2t−30∘−tan211)

 the total response

y(t)=10122sin(t+60∘−tan−1111)−45cos(2t−30∘−tan−1211)

核心能力二 狀態空間之控制系統

  - 系統之可控制性

狀態空間設計(State space design)

古典控制

U(s)              Y(s)

轉移函數

G(s)

輸入

輸出

G(s)=Y(s)U(s)

近代控制

X=Ax+BuY=Cx+Du

U(s)                Y(s)

X:狀態變數x=x1x2⋮A∈Rn×nB∈Rn×1C∈R1×nD∈R

例題:	y(t)

M

k

β

Μy(t)+By(t)+ky(t)=u

轉移函數法：

(MS2+BS+k)Y(s)=U(s)∴Y(s)U(s)=1MS2+BS+k

狀態空間法：

令y(t)=x1(t)    ,    {y(t)=x2(t)(controllable canonical form)

x1=x2x2=y=−kMy-BMy+1Mu          =−kMx1−BMx2+1Mu⇒x1

狀態空間之控制器設計(Controller Design in State Space)

可控制性(Controllability)

x=Ax+Buy=Cxx=x1x2⋮若存在u=−kx=−k1k2…knx1x2⋮

使得x=(A−Bk)x其中A−Bk之特徵值得以任意決定，則稱此系統可控制，u為系統之狀態回授(statefeedback)控制器

條件：

控制性矩陣

Qc=BABA2B⋯An−1B

若 Rank(Qc)=n則系統可控制或Qc≠0亦可為判斷依據

  - 何謂狀態回授控制?

控制器設計

x=Ax+Buu=−kx=−k1k2…knx1x2⋮

- 先決定控制閉迴路系統之理想極點或理想特徵值

- λ(A−Bk)=理想值

求出適當k值

例題1: 一控制系統其狀態方程式如下:

如果控制器設計為

其中a、b 為常數。當此閉迴路控制系統為穩定時，求出a、b之範圍並以a為橫軸，b為縱軸，畫出a-b之關係圖。

例題2:

無摩擦

y(t)

m=1

- 定義狀態變數 x1=y,x2=y 以及輸出 y , 列出狀態方程式。

- 設計狀態回授控制器 u=2r−k1k2x1x2 使得閉迴路特徵值為−1±j

- 將此系統以下列方塊圖表示

r

y

+-               +-

Sol:

(1)

md2y(t)dt2=u(t)⋅m=1y=u,∵x1=y,x2=yx1=x2x2=u⇒x1

(2)

u=2r−k1k2x1代入

x1

λ−1

(3)

u=2r−2x1−2x22(r−x1)−2x2

r

2

y

2S

u

1S2

+	+

-	-

例題3:

Give a system G(s)=4s2+25s+38s3+9s2+26s+24, if the state space representation of the system is x=−2000a0    求a,b,g?

Sol:

G(s)=4s2+25s+38s3+9s2+26s+24As+2+Bs−a+Cs−b

s3+9s2+26s+24=(s+2)(s+3)(s+4)a=−3,b=−4A=2B=1C=1g=21

例題4:

例:

V為輸入電壓，VC為輸出電壓

求狀態方程式及轉移函數

Sol：

電路方程式為

Ldidt+Ri+VC=V

轉移函數法(兩邊取拉式轉換)

(LS+R)I(s)+VC(s)=V(s)CSVC(s)=I(s)(LS+R)CS+1VC(s)=V(s)VC(s)V(s)=1CS(LS+R)+1

狀態空間法

令VC=x1    i=x2

x1=1cx2x2=−1Lx1−RLx2+1LV⇒x1